Cloud-Based Geo-Fencing System for Pipeline Robot (Prototype)

1. Objective

The objective of this R&D is to design and validate a cloud-based geo-fencing system for a pipeline inspection robot. The system tracks the robot’s location using GPS, sends location data to a cloud platform over LTE, evaluates geo-fence conditions in the cloud, and sends control commands back to the robot based on location.

This document focuses on feasibility, design decisions, hardware selection, and system assumptions for a working prototype (POC).

2. Scope of the System

Included in Scope

- GPS-based location tracking

- LTE-based communication with cloud

- MQTT protocol for data exchange

- Cloud-side geo-fence evaluation

- Command reception at device level

Excluded from Scope

- Mechanical tripper design

- HoMoSeP internal actuation design

- Safety-critical real-time control

- Advanced localization methods (IMU, odometry, RTK GPS)

3. System Overview

The system consists of an embedded device mounted on a robot, connected to a GPS module and an LTE modem. The device periodically sends GPS coordinates to a cloud server. The cloud checks whether the received coordinates fall within a predefined geo-fence. Based on this evaluation, the cloud sends control commands back to the device.

The embedded device acts as a data publisher and command executor, while all geo-fence intelligence resides in the cloud.

4. Design Assumptions

- Continuous LTE connectivity is available during operation

- GPS signal availability is sufficient for prototype demonstration

- Cloud latency of a few seconds is acceptable

- System is intended for monitoring and controlled triggering, not life-critical safety actions

These assumptions are explicitly acknowledged as prototype-level constraints.

5. R&D Analysis and Design Decisions

5.1 Location Tracking Technology

Technology Chosen: GPS (Global Positioning System)

Reasoning:

- Provides direct latitude and longitude

- Widely supported and low cost

- UART-based communication simplifies integration

Limitations:

- Requires open or semi-open sky

- Reduced reliability inside pipelines

Decision: GPS is suitable for prototype validation. Signal loss is treated as a known limitation.

5.2 Communication Technology

Technology Chosen: LTE (4G) using Quectel EC25 / EG25

Reasoning:

- Wide-area coverage

- Reliable cloud connectivity

- Industry-standard module with AT command interface

Decision: LTE is selected as the primary communication channel between device and cloud.

5.3 Communication Protocol

Protocol Chosen: MQTT

Reasoning:

- Lightweight and efficient for embedded systems

- Supports publish/subscribe model

- Enables two-way communication

Data Format: JSON

Decision: MQTT over LTE is finalized for all device–cloud communication.

5.3.1 Controller and LTE Integration Update

Original Approach:

- ESP32 as microcontroller

- Separate LTE module (Quectel EC25 / EG25)

Updated Approach:

- 7SEMI EC200U LTE 4G module with integrated STM32 MCU

Reasoning:

- Combines microcontroller and LTE modem on a single board

- Reduces hardware count from two boards to one

- Simplifies UART wiring and power management

- Improves system reliability for prototype

Decision: The ESP32 and separate LTE module are replaced with a single 7SEMI EC200U LTE 4G STM32-based module.

5.4 Geo-Fencing Logic Placement

Approach Selected: Cloud-based geo-fencing only

Explanation:

- Device sends raw GPS data

- Cloud compares GPS data with stored geo-fence boundaries

- Cloud decides and sends command

Advantages:

- Simplifies embedded firmware

- Centralized control and configuration

Limitations:

- Depends entirely on network availability

Decision: All geo-fence evaluation logic resides in the cloud.

5.5 Data Transmission Rate

Initial Concept: 500 ms updates

R&D Finding:

- GPS updates typically occur at 1 Hz

- Higher transmission rate causes duplicate data and unnecessary power usage

Decision:

- Prototype transmission rate set to 5 seconds

- Can be optimized later if required

5.6 Event-Based Trigger (Button)

Purpose: Manual or emergency location reporting

Behavior:

- On button press, device sends immediate GPS update

- Message sent using a dedicated MQTT topic

Decision: Button-based event messaging is included but not mandatory for MVP.

6. System Data Flow

- GPS module provides latitude and longitude to ESP32

- ESP32 publishes GPS data to cloud via LTE using MQTT

- Cloud checks geo-fence boundaries

- Cloud publishes control command based on geo-fence result

- ESP32 receives command and triggers corresponding logic

7. Hardware Requirements

7.1 Core Hardware

- 7SEMI EC200U LTE 4G module with integrated STM32 MCU

- GPS Module (u-blox NEO-6M / equivalent)

- LTE-enabled SIM card with data plan

- GPS antenna

- LTE antenna

- 5V / 3A regulated power supply

7.2 Development and Support Hardware

- USB cable for programming and debugging

- Push button (optional, for event trigger)

- Jumper wires / connectors

8. Minimum Viable Prototype (MVP)

The MVP is considered successful if:

- GPS data is received by the ESP32

- GPS data is published to the cloud

- Cloud dashboard displays location

- Cloud sends a command

- Device receives and logs the command

Geo-fence logic accuracy and actuation are secondary to connectivity validation in MVP.

9. Risks and Mitigations

| Risk | Impact | Mitigation |
| --- | --- | --- |
| GPS signal loss | No location updates | Handle NO FIX state |
| LTE instability | No geo-fence decision | Retry and reconnect logic |
| High power draw | System resets | Adequate power supply |
| Cloud latency | Delayed trigger | Accepted prototype limitation |

10. Conclusion

This R&D confirms that a cloud-based geo-fencing system using GPS, LTE, and MQTT is technically feasible and suitable for a prototype pipeline robot application. The design prioritizes simplicity on the embedded device and centralized decision-making in the cloud. All major technical risks are understood and acceptable within prototype constraints.